Copyright 2013 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

This Product MasterSpec Section is licensed by ARCOM to Avanti Systems, Inc. ("Licensee").

This Product MasterSpec Section modifies the original MasterSpec text and does not include the full content of the original MasterSpec Section.

Revisions made to the original MasterSpec text are made solely by the Licensee and are not endorsed by, or representative of the opinions of, ARCOM or The American Institute of Architects (AIA). Neither AIA nor ARCOM are liable in any way for such revisions or for the use of this Product MasterSpec Section by any end user. A qualified design professional should review and edit the document to suit project requirements.

For more information, contact Avanti Systems, Inc., 200 Pemberwick Rd., Greenwich, CT 06831; Phone: (877) 282-6843; Fax: (914) 934-5992; Website: www.avantisystemsusa.com; Email: sales@avantisystemsusa.com.

For information about MasterSpec, contact ARCOM at (800) 424-5080 or visit www.MasterSpec.com.

SECTION 083213 - SLIDING ALUMINUM-FRAMED GLASS DOORS

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

Avanti Systems products are intended for interior use ONLY.

- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section includes aluminum-framed partition systems with sliding glass doors for interior locations.

        - Related Requirements:

Retain subparagraph below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 084113 "Aluminum-Framed Entrances and Storefronts" for coordinating finish among aluminum framing.

      - PREINSTALLATION MEETINGS

Retain "Preinstallation Conference" Paragraph below if Work of this Section is extensive or complex enough to justify a conference.

        - Preinstallation Conference: Conduct conference at [Project site] <Insert location>.

If needed, insert list of conference participants not mentioned in Section 013100 "Project Management and Coordination."

      - ACTION SUBMITTALS

        - Product Data: For each type of product.

          - Include construction details, material descriptions, fabrication methods, dimensions of individual components and profiles, hardware, finishes, and operating instructions.

        - Shop Drawings: For aluminum-framed partition systems with sliding glass doors.

          - Include plans, elevations, sections, and details.

          - Detail attachments to other work, and between units, if any.

          - Include hardware and required clearances.

        - Samples for Initial Selection: For each type of sliding aluminum-framed glass door indicated.

          - Include Samples of hardware and accessories involving color selection.

        - Product Schedule: For sliding aluminum-framed glass doors.[ Use same designations indicated on Drawings.]

      - INFORMATIONAL SUBMITTALS

Coordinate "Qualification Data" Paragraph below with qualification requirements in Section 014000 "Quality Requirements" and as may be supplemented in "Quality Assurance" Article.

        - Qualification Data: For Installer.

        - Sample Warranty: For manufacturer's warranty.

      - CLOSEOUT SUBMITTALS

        - Maintenance Data: For finishes, operable panels, and operating hardware to include in maintenance manuals.

      - QUALITY ASSURANCE

        - Manufacturer Qualifications: A manufacturer capable of fabricating sliding aluminum-framed partition systems that meet or exceed performance requirements indicated and of documenting this performance by inclusion in lists and by labels, test reports, and calculations.

        - Installer Qualifications: An installer acceptable to sliding aluminum-framed partition systems manufacturer for installation of units required for this Project.

      - WARRANTY

When warranties are required, verify with Owner's counsel that warranties stated in this article are not less than remedies available to Owner under prevailing local laws.

        - Manufacturer's Warranty: Manufacturer agrees to repair or replace components of sliding aluminum-framed partition system that fail in materials or workmanship within specified warranty period.

          - Failures include, but are not limited to, the following:

            - Failure to meet performance requirements.

            - Failure of laminated glass.

Verify available warranties and warranty periods.

          - Warranty Period:

            - Sliding Door: Two years from date of Substantial Completion.

Warranty period for glazing units varies by type. Some manufacturers may insist that warranties begin on date of manufacture or sales.

            - Laminated Glass: Two years from date of Substantial Completion.

Warranty period for aluminum finish depends on type.

            - Aluminum Finish: Two years from date of Substantial Completion.

- PRODUCTS

      - MANUFACTURERS

Manufacturers and products listed in SpecAgent and MasterWorks Paragraph Builder are neither recommended nor endorsed by the AIA or ARCOM. Before inserting names, verify that manufacturers and products listed there comply with requirements retained or revised in descriptions and are both available and suitable for the intended applications. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

Retain "Basis-of-Design Product" Paragraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option(s) if manufacturer's name and model number are indicated on Drawings.

        - Basis-of-Design Product: Subject to compliance with requirements, provide Avanti Systems, Inc.; [Barn Sliding Glass Door] [Cloud Assisted Sliding Glass Door] [Pocket Sliding Glass Door] or comparable product by one of the following:

          - DORMA USA, Inc.

          - Nana Wall Systems, Inc.

          - <Insert manufacturer's name>.

        - Source Limitations: Obtain aluminum-framed partition system with sliding glass doors from single source from single manufacturer.

      - PERFORMANCE REQUIREMENTS

Retain "Sound Transmission Class (STC)" Paragraph below after verifying availability of test data. STC evaluates construction subject to interior, spoken-voice frequencies.

        - Sound Transmission Class (STC): Rated for not less than [28] [32] STC when tested for laboratory sound transmission loss according to ASTM E 90 and determined by ASTM E 413.

      - GLAZING

        - Glass and Glazing: Manufacturer's standard glazing system.

Retain "Safety Glazing Labeling" Subparagraph below if applicable and if labeling is required. Not all manufacturers participate in third-party testing programs.

          - Safety Glazing Labeling: Permanently mark safety glazing with certification label of [the SGCC] [the SGCC or another certification agency acceptable to authorities having jurisdiction] [or] [the manufacturer]. Label shall indicate manufacturer's name, type of glass, thickness, and safety glazing standard with which glass complies.

          - Tint: [Clear] [Gray] [Bronze] [Green] <Insert tint>.

      - HARDWARE

Some manufacturers may offer door pulls in other designs and materials. Revise "Door Pulls" Paragraph below if selecting non-standard pull.

        - Door Pulls: Provide manufacturer's standard pull.

          - Color and Finish: [Clear-anodized aluminum] [RAL 9006 white aluminum, polyester powder coat] [As indicated by manufacturer's designations] [Match door frame] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color and finish>.

Revise "Lock" Paragraph below to suit Project's locking requirements, or insert others. Multipoint locking devices improve security and performance but are not offered by all manufacturers.

        - Lock: Install manufacturer's cylinder lock and [multipoint ]locking device on each movable panel, lockable from the inside [only] [and outside]. Adjust locking device to allow unobstructed movement of the panel across adjacent panel in the direction indicated.

          - Floor Lock: [Thumb-turn] [Key] actuated.

          - 60-inch- (1524-mm-) high, locking, ladder-pull handle.

      - ACCESSORIES

        - Fasteners: Noncorrosive and compatible with door members, trim, hardware, anchors, and other components.

Generally, retain "Exposed Fasteners" Subparagraph below. Revise if exposed fasteners are permitted.

          - Exposed Fasteners: Do not use exposed fasteners to the greatest extent possible. For application of hardware, use fasteners that match finish hardware being fastened.

        - Anchors, Clips, and Accessories: Provide anchors, clips, and accessories of aluminum, nonmagnetic stainless steel, or zinc-coated steel or iron for sliding glass doors.

      - FABRICATION

        - Fabricate sliding aluminum-framed glass doors in sizes indicated. Include a complete system for assembling components and anchoring doors.

      - GENERAL FINISH REQUIREMENTS

        - Comply with NAAMM's "Metal Finishes Manual for Architectural and Metal Products" for recommendations for applying and designating finishes.

        - Protect mechanical finishes on exposed surfaces from damage by applying a strippable, temporary protective covering before shipping.

        - Appearance of Finished Work: Variations in appearance of abutting or adjacent pieces are acceptable if they are within one-half of the range of approved Samples. Noticeable variations in the same piece are not acceptable. Variations in appearance of other components are acceptable if they are within the range of approved Samples and are assembled or installed to minimize contrast.

      - ALUMINUM FINISHES

        - Clear Anodic Finish: AAMA 611, AA-M12C22A31, Class II, 0.010 mm or thicker.

"Powder-Coat Finish" Paragraph below references AAMA standard for pigmented organic coating on extrusions and panels.

        - Powder-Coat Finish: AAMA 2603 except with a minimum dry film thickness of 1.5 mils (0.04 mm). Comply with coating manufacturer's written instructions for cleaning, conversion coating, and applying and baking finish.

          - Color and Gloss: [RAL 9006 white aluminum, polyester powder coat] [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color and gloss>.

- EXECUTION

      - EXAMINATION

        - Examine openings, substrates, structural support, anchorage, and conditions, with Installer present, for compliance with requirements for installation tolerances and other conditions affecting performance of the Work.

        - Verify rough opening dimensions, levelness of threshold substrate, and operational clearances.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION

        - Comply with Drawings, Shop Drawings, and manufacturer's written instructions for installing doors, hardware, accessories, and other components.

        - Install sliding glass doors level, plumb, square, true to line, without distortion, without warp or rack of frames and panels; anchored securely in place to structural support; and in proper relation to other adjacent construction.

      - ADJUSTING, CLEANING, AND PROTECTION

        - Lubricate hardware and moving parts.

        - Adjust operating panels to provide a tight fit at contact points for smooth operation, without binding. Adjust hardware for proper alignment, smooth operation, and proper latching without unnecessary force or excessive clearance.

        - Clean exposed surfaces immediately after installing sliding aluminum-framed glass doors. Avoid damaging protective coatings and finishes. Remove nonpermanent labels, excess sealants, glazing materials, dirt, and other substances.

        - Remove and replace glass that has been broken, chipped, cracked, abraded, or damaged during construction period.

        - Protect sliding glass door surfaces from contact with contaminating substances resulting from construction operations. If contaminating substances contact sliding aluminum-framed glass door surfaces, remove contaminants immediately according to manufacturer's written instructions.

        - Replace damaged components.

END OF SECTION 083213